AU-A220 FW Update Procedure

1. Unplug the PSU

2. Press and hold ‘MUTE’ Key

3. Plug in the PSU and plug RS232 between PC and Unit. All LEDs will illumniate. The ‘PW’ LED will flash.

4. Open ‘CDPS 2016_V0.4K.exe’

5. Click ‘Search’ button

6. Click the device which you found

7. Click ‘System’ Tab

8. open the file and update

9. Click program icon

10. System will reboot automatically

Release Note:

| Date | Version | Note |
| --- | --- | --- |
| 2017.08.01 | V0.12 | Initial Test |